Multi-Functional Cane

ELBC803- Embedded Design Project

By

Ali Yazbeck – 94764

Serena Haidar – 90457

Zein Attar - 94266

Submitted to the Faculty of Engineering of the

Islamic University in Lebanon

Baalbek, Lebanon

In partial fulfillment of the requirements for the degree of

BACHELOR OF ENGINEERING IN COMPUTER AND COMMUNICATIONS

Spring 2024-2025

Approved by:
Dr. Mohammad Chreif

TABLE OF CONTENTS

CHAPTER 1	INTRODUCTION	1

1.	What Is Engineering?	1

2.	Project Idea and Objectives	1

3.	What Is the Arduino Mega 2560?	2

CHAPTER 2	System Design and Implementation	4

2.1	Introduction	4

2.2	Required Components	4

2.3	Circuit Design	5

2.4	Implementation Steps	6

2.4.1 LDR Sensor	6

2.4.2 GPS+GSM modules	7

2.4.3 Ultrasonic Sensor and Audio Feedback	8

2.4.4 Water sensor with buzzer	10

2.4.5. Rechargeable Li-ion batteries	12

2.5 Hardware	13

CHAPTER 3	Conclusion and Future Work	14

3.1	Conclusion	14

3.2	Future Work	14

CHAPTER 4	References	15

‘

LIST OF FIGURES

Figure ‎2-1 Complete circuit	5

Figure ‎2-2 Circuit	5

Figure ‎2-3 Led connections	6

Figure ‎2-4 Connecting Arduino to breadboard	6

# INTRODUCTION

## What Is Engineering?

Engineering is the disciplined art of applying scientific and mathematical principles to design, build, and optimize structures, systems, and processes for humanity’s benefit [1]. Fundamentally, engineering solves real-world problems, serving people and the planet alike, and propelling society forward [2].

The field branches into specialties such as biomedical, chemical, electrical, civil, materials, and mechanical engineering, each addressing unique challenges from health to infrastructure to energy. Beyond these fields, emerging and interdisciplinary domains, like environmental, aerospace, software, and mechatronics, illustrate engineering’s boundless reach across modern life [3].

Yet all engineers share a common framework: define requirements, model solutions, test prototypes, and refine designs until form and function unite seamlessly.

## Project Idea and Objectives

Globally, over 2.2 billion people live with a near or distance vision impairment or blindness. Alarmingly, at least 1 billion of these cases are treatable or preventable, highlighting a profound need for assistive technologies [4]

Vision loss often leads to reduced independence, limited mobility, and an increased risk of accidents, especially when navigating new environments.

Today, existing mobility aids like traditional white canes, often rely on mechanical contact feedback. More advanced versions offer vibration alerts or audio cues, yet neglect hazards like water puddles, low-light visibility, or emergency signaling. These limitations leave visually impaired people vulnerable in critical situations.

To bridge this gap, our project presents a Multifunctional Smart Cane: an integrated assistive system designed to extend the capabilities of traditional canes using embedded technology.

Key features:

- Ultrasonic sensors to detect nearby solid obstacles

- A water sensor to identify slippery surfaces like puddles

- A GPS + GSM module to allow users to send location-tagged SOS messages in emergency cases

- An LDR-based lighting system that activates a bright LED in dark environments to increase user visibility

- A digital compass and sound module intended to provide directional orientation cues

- A rechargeable Li-ion battery system, ensuring portable, long-lasting operation.

Project Objectives:

- Detect obstacles and slippery surfaces: Using ultrasonic sensors for solid objects and a water sensor for puddles.

- Enable Emergency response: One-button system sends GPS coordinates via SMS using GSM module.

- Improve visibility in low-light: Automatic LED activation triggered by an LDR sensor to make the user visible to others.

- Assist with basic directional navigation: A digital compass paired with sound feedback to help orient the user.

- Ensure long-lasting operation: Powered by a rechargeable Li-ion battery for portability and efficiency.

By delivering these features, we address the fundamental challenges of safety, independence, and confidence for visually impaired users.

## What Is the Arduino Mega 2560?

The Arduino Mega 2560 is a high-performance microcontroller board built around the ATmega2560 chip. It is designed for projects that require numerous inputs and outputs or complex control systems. With 54 digital I/O pins, 16 analog inputs, and 4 serial communication ports, it is well-suited for advanced embedded and automation projects, robotics, and large-scale sensor networks. [5]

Unlike boards with integrated wireless capabilities, such as the ESP32, the Mega 2560 relies on external modules (e.g., Bluetooth, GSM, or Wi-Fi) to extend its connectivity. It remains a popular choice for developers who prioritize pin availability and processing stability in large projects.

Arduino Mega 2560 Specifications [6]

- Microcontroller: ATmega2560

- Operating Voltage: 5V

- Input Voltage (recommended): 7–12V

- Input Voltage (limits): 6–20V

Memory

- Flash Memory: 256 KB (8 KB used by the bootloader)

- SRAM: 8 KB

- EEPROM: 4 KB

Digital and Analog I/O

- Digital I/O Pins: 54 (15 with PWM output capability)

- Analog Input Pins: 16

- PWM Channels: 15

- UARTs: 4 hardware serial ports

- Interfaces Supported: SPI, I2C, USB

Clocks and Timers

- Clock Speed: 16 MHz (external crystal oscillator)

- Timers: Includes hardware timers for PWM, timing, and interrupts

Others Features

- ICSP header for direct microcontroller programming

- Reset button, voltage regulator, and power jack

- Compatible with standard Arduino shields and the Arduino IDE

# System Design and Implementation

## Introduction

In this chapter, the components used in the Multi-Functional Smart Cane are presented, followed by a description of the systematic steps taken for its implementation.

The primary objective of this system is to enable visually impaired individuals to navigate their surroundings safely and independently by integrating sensors and modules for obstacle detection, location tracking, darkness detection, and emergency messaging.

## Required Components

The following table lists all the components used to build the multi-functional smart cane. Each part was chosen based on its compatibility with the Arduino Mega 2560 and its relevance to the features we wanted to implement.

|  | ARDUINO MEGA-2560 x1 The brain of the system. |
| --- | --- |
|  | Breadboard x1 For connecting components |
|  | LDR Sensor x1 Detects light intensity to automatically turn on a LED when it’s dark |
|  | LED Light Strip x1 Lights up the path in dark areas based on the LDR readings |
|  | Resistor 220-ohm x7 To implement electrical resistance as a circuit element |
|  | Slide Switch, 6-pin ON_ON x1 To power the cane ON and OFF |
|  | Ultrasonic sensor 412 CH x3 To detect obstacles and send feedback to the DFPlayer |
|  | GPS MODULE NEO-6MV2 x1 To acquire the location from satellites |
|  | GSM SIM800L Module x1 To send an SMS when the emergency button is pressed |
|  | Tactile switch 4-pin x1 Serves as an emergency button. When pressed, it activates the GSM module which sends the emergency SMS |
|  | DFPlayer module X1 kjklweflk |
|  | Micro SD Card x1 To store the voice messages used for navigation feedback |
|  | Mini Speaker 4ohm, 3W x1 To play the sounds stored in the microSD card |
|  | Compass digital sensor x1 To orient the user |
|  | Li-ion 18650 2200mAh TAG battery, 3.7V x2 To charge the cane |

## Circuit Design

Figure 2.1 shows the full circuit schematic of the system, which includes the Arduino Mega 2560 microcontroller, ultrasonic sensors, a GPS and GSM modules for emergency functionality, and other components connected with proper power management and grounding. ADD THE LDR SENSOR + LAMP + MISSING PARTS. IMAGE TO BE REPLACED

Figure ‎2.1 Full System Circuit Design

## Implementation Steps

### 2.4.1 LDR Sensor

In this step, an LDR sensor is connected to the analog pin A0 and a LED to the digital pin A8 of the Arduino Mega 2560. When the LDR detects darkness (light level gets below a set threshold), the Arduino turns on the LED automatically. This feature helps the user to walk safely in low-light environments by turning on the light without any manual input.

Figure ‎2.2 LDR and LED light circuit connected to Arduino Mega 2560

Code:

int ldrPin = A0;       // LDR connected to analog pin A0

int ledPin = 8;        // LED connected to digital pin 8

int threshold = 500;   // Light threshold for darkness

void setup() {

pinMode(ledPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

int lightLevel = analogRead(ldrPin); // Read light value

if (lightLevel < threshold) {

digitalWrite(ledPin, HIGH);        // Turn on LED

} else {

digitalWrite(ledPin, LOW);         // Turn off LED

} }

Explanation

- analogRead(): reads the light level from the LDR.

- If the value is less than 500 (dark), the LED turns on.

- If it's brighter, the LED stays off.

This ensures safer navigation for the user in the dark, making them more visible to others.

### 2.4.2 GPS+GSM modules

To assist the visually impaired person in case of an emergency, such as a fall, an injury, or getting lost, an emergency button is implemented. When pressed, it sends the person’s location coordinates in readable format, as a text message to a preset emergency contact.

The location is acquired from satellites using the GPS module, and the GSM SIM module send the message to a phone number.

Wiring Overview:

The GPS and GSM modules both have 4 pins : TX, RX, VCC and GND.

- GSM module(SIM800L GSM module):

  - VCC → 5V power supply

  - GND → Ground

  - TX → Connected to pin RX2(pin 17) on Arduino Mega

  - RX → Connected to pin TX2 (pin 16) on Arduino Mega

- GPS module(NEO-6M GPS):

  - VCC → 5V power supply

  - GND → Ground

  - TX → Connected to pin RX1 /ADx (pin 2x) on Arduino Mega

  - RX → Not connected, since the GPS only sends and doesn’t receive information.

// insert code screenshot with some explanation to how the components work

### 2.4.3 Ultrasonic Sensor and Audio Feedback

In order to detect obstacles and avoid them, the cane needs to be equipped with ultrasonic sensors and provide immediate audio feedback.

The Arduino Mega reads input from the sensors, and when an obstacle is close, it activates the DFD Player Mini, which outputs voice messages stored on an SD card, guiding the user left or right.

For simplicity in the simulation phase, a buzzer feedback is implemented, and the distance controlled by a potentiometer.

This feedback mechanism operates continuously, ensuring that the user is warned of obstacles in real time

Figure 3:Ultrasonic sensor & buzzer wirings in Proteus

Wiring Overview:

The ultrasonic sensor has 5 main pins: Trigger, Echo, TestPin, VCC and GND.

- Ultrasonic sensor (HC-SR04):

  - VCC → 5V power supply

  - GND → Ground

  - Trigger → Connected to pin PA3 / AD3 (pin 25) on Arduino Mega

  - Echo → Connected to pin PA5 / AD5 (pin 27) on Arduino Mega

  - TestPin → Connected to the wiper of a potentiometer (RV1) to simulate distance changes

- Potentiometer (RV1):

  - One end to 5V

  - Other end to GND

  - Wiper (middle pin) → Connected to TestPin of the ultrasonic sensor, allowing the simulation of different distances by adjusting the potentiometer, mimicking how objects are positioned closer or farther.

- Buzzer:

  - Positive pin → Connected to pin PA4 / AD4 (pin 26) on Arduino Mega

  - Negative pin → Connected to GND

Code:

const int trigPin = 25; // Ultrasonic Trigger pin

const int echoPin = 27; // Ultrasonic Echo pin

const int buzzerPin = 26; // Buzzer control pin

void setup () {

pinMode (trigPin, OUTPUT);

pinMode (echoPin, INPUT);

pinMode (buzzerPin, OUTPUT);

Serial.begin(9600);

}

void loop () {

// Send ultrasonic pulse

digitalWrite (trigPin, LOW);

delayMicroseconds (2);

digitalWrite (trigPin, HIGH);

delayMicroseconds (10);

digitalWrite (trigPin, LOW);

// Read echo response time

long duration = pulseIn (echoPin, HIGH);

// Convert to distance (cm)

float distance = duration * 0.034 / 2;

// Activate buzzer if object is too close

if (distance > 0 && distance < 15) {

digitalWrite (buzzerPin, HIGH);

} else {

digitalWrite (buzzerPin, LOW);

}

delay (300); // Stabilize readings

The code continuously measures distance using the ultrasonic sensor, for real-time obstacle detection. It sends a short pulse, waits for the echo, and calculates the distance based on the time it takes to return.

- If the measured distance is less than 15 cm, it means that an object is near, which turns ON the buzzer to alert the user.

- If no nearby object is detected, the buzzer remains OFF.

### 2.4.4 Water sensor with buzzer

To avoid slipping on water puddles on the ground or wet surfaces, a water detection feature is implemented, with a water sensor and a buzzer. The water sensor + how it works.

To simulate this, a logic probe is used to detect water, providing a simple yet effective feedback system.

The signal pin outputs HIGH when water is detected, and the Arduino activates the buzzer in response, sharing the same pin logic used for obstacle detection.

Wiring overview:

- Water sensor module:

This module has one main pin connecting to the Arduino Mega to signal the presence of water.

  - VCC pin → Connected to 5V power supply

  - GND pin → Connected to Ground

  - S (Signal) pin → Connected to PA6 / AD6 (pin 28) on Arduino Mega

- Logic probe:

  - Connected to the signal pin S of the water sensor

  - Logic 1 (HIGH) → simulates the presence of water.

  - Logic 0 (LOW) → simulates dry conditions (no water detected).

Code:

const int waterSensorPin = 28; // PA6 / Analog pin 6

const int buzzerPin = 26; // Buzzer control pin    // PA7 / Analog pin 7

void setup () {

pinMode (buzzerPin, OUTPUT);

pinMode (waterSensorPin, INPUT); // Read water sensor signal (S pin)

Serial.begin(9600);

}

void loop () {

int waterState = digitalRead(waterSensorPin);

if (waterState == HIGH) {// LOW means water is detected

Serial.println("⚠️ Water Detected!");

digitalWrite (buzzerPin, HIGH); // Turn ON buzzer

} else {

Serial.println("✅ No Water");

digitalWrite (buzzerPin, LOW); // Turn OFF buzzer

}

delay	(300); // Wait a bit

}

The code checks for the presence of water using a water sensor. It reads the S pin of the sensor, which outputs a digital signal: HIGH if water is detected and LOW if the surface is dry (based on how you wired the logic probe in simulation).

If water is detected (signal is HIGH), the system:

- Prints a warning message: "Water Detected!"

- Activates the buzzer to alert the user.

This check happens repeatedly in the loop, so the system can immediately respond whenever water appears on the ground.

### 2.4.5. Rechargeable Li-ion batteries

Since the cane is a mobile assistive device, it needs a power source of its own to function optimally for long hours. Two lithium-ion rechargeable batteries provide a constant power of 3.3V, enough to activate all of the components and features of the cane. Additionally, an ON/OFF button for the cane can deactivate all components when the individual is not using it, ensuring power is saved.

In simulation, a DC power supply block was used instead of batteries to provide stable voltage input during testing in Proteus.

overcharge, or deep discharge protection.

## 2.5 Hardware

INCLUDE IMAGE OF HARDWARE

This section explains the real-world assembly and integration of all hardware components into a fully functional prototype.

After simulating the system on Proteus 8, the multi-functional cane was physically assembled using sensors, modules, an Arduino Mega 2560 microcontroller, a breadboard, and jumper wires. All modules were tested individually then incorporated into the system. The goal was to create a fully operational assistive device for visually impaired individuals.

2.5.1 Physical Assembly( (LATER WHEN WE ASSEMBLE IT)

All components were mounted onto or near the cane structure using adhesive supports, a small project box, and zip ties for stability. A solderless breadboard was used for temporary connections during development, while key sensors and modules were directly connected to the Arduino Mega 2560 using jumper wires. The Arduino board was securely housed near the top of the cane, allowing easy access to the USB port for power and code updates.

- 🧩 2.5.2 Power Management

Purpose: Show how you handled powering the system.

Write about:

- Number and type of Li-ion batteries used

- Voltage regulation (if used, e.g., AMS1117, buck converter)

- ON/OFF switch placement and function

- Charging setup (USB, external charger, TP4056 module, etc.)

📝 Tip: Include total power consumption estimate, if known.

Include:

- Battery specs

- Voltage regulation (if applicable)

- Simulation replacement with DC power block

Example:

“The cane is powered by two 3.7V Li-ion rechargeable batteries connected in series with an ON/OFF switch. In the Proteus simulation, a regulated DC power supply of 5V was used instead.”

The system is powered using two rechargeable 3.7V Li-ion batteries connected in series to provide sufficient voltage for all components. A slide ON/OFF switch was added between the power source and the Arduino to allow the user to easily turn the device on or off. To ensure safe and regulated operation, the voltage from the batteries was monitored, and a 5V power rail was established to supply the sensors and modules. This mobile power solution ensures the cane can operate independently for several hours.

- 2.5.3 Sensor and Module Integration

Purpose: Explain how you physically connected each major sensor.

Break this into short bullet points:

- LDR Sensor + LED: Soldered directly or placed on a mini breadboard; connected to A0 and D13.

- Ultrasonic Sensor: Mounted on the cane tip, connected to pins 25 and 27.

- Buzzer: Attached near the grip, wired to pin 26.

- Water Sensor: Placed near bottom of cane, connected to pin 28.

- GPS Module: Connected to Serial1 (pins 18/19); mounted on top for better reception.

- GSM Module: Connected to Serial2 (pins 16/17); antenna placed externally.

- Emergency Button: Attached to grip, wired to pin 7.

📝 Mention any shielding, shrink tubing, or glue used to fix parts.

Example

The smart cane integrates multiple sensors and modules, each with a dedicated function:

- LDR Sensor and LED: The LDR was connected to analog pin A0 to detect ambient light. A white LED was connected to digital pin 13 and placed near the bottom of the cane to automatically illuminate in dark environments.

- Ultrasonic Sensor: The HC-SR04 sensor was mounted at the front of the cane and connected to pins 25 (Trig) and 27 (Echo). It detects obstacles ahead and sends distance measurements to the Arduino.

- Buzzer: A small piezo buzzer was mounted near the cane handle, connected to pin 26. It provides auditory alerts when an obstacle or water is detected.

- Water Sensor: Placed near the base of the cane to detect water on the ground, the sensor’s signal (S) pin was connected to pin 28.

- GPS Module: The GPS module was connected to the Arduino’s Serial1 (pins 18 and 19) to receive live location data. It was placed near the top of the cane for optimal satellite signal reception.

- GSM Module: The SIM800L GSM module was wired to Serial2 (pins 16 and 17) to send SMS alerts. Its antenna was extended outward to ensure network coverage.

- Emergency Button: A tactile pushbutton was installed near the handle for quick access. It was connected to digital pin 7, and pressing it triggers an SMS message with the user’s coordinates.

All modules shared a common ground and were powered via the Arduino’s 5V output or directly from the Li-ion batteries through a regulated line.

- 🧪 2.5.3 Testing Under real conditions

Purpose: Describe how you validated the prototype in real life.

Write about:

- Testing in a dark room (LDR)

- Obstacle detection with real walls/objects (ultrasonic)

- Placing water on the floor to test water sensor

- Outdoor GPS fix and receiving SMS on a phone

- Response time and buzzer loudness

📝 Mention who tested it (you, a peer, etc.), and any challenges faced like noise, delays, weak signals...

Brief mention of how you tested:

- LDR with slider

- Water sensor with logic probe

- GPS via injected NMEA sentences

- SMS monitored via virtual terminal

Example:
The fully assembled cane was tested in various real-world scenarios to evaluate its functionality:

- LDR/LED: The LED activated in dim lighting environments and turned off under normal room lighting, verifying LDR response accuracy.

- Obstacle Detection: The ultrasonic sensor reliably detected nearby walls and objects within 15 cm, activating the buzzer to alert the user.

- Water Detection: When placed near water, the sensor successfully triggered the buzzer alert, helping simulate wet surfaces or puddles.

- GPS and GSM: In open outdoor areas, the GPS acquired valid satellite data after a short delay. Pressing the emergency button sent an SMS with the current latitude and longitude to a predefined phone number.

- System Behavior: The system correctly handled multiple inputs, such as simultaneous darkness and obstacle detection, without errors.

Figure ‎2-3 Connecting Arduino to breadboard

# Conclusion and Future Work

## Conclusion

In this paragraph, you write a summary of what you presented in the above two chapters. Instead of using expressions like “in our project we will implement ….”, in the conclusion we usually use expressions like “As we could see above, in our project we implemented ……”.

Besides, you need to write about the experiences you learned while taking the course and implementing the project.

## Future Work

In this paragraph, you need to write any idea you have about expanding the work you did in your project to be more developed and to be used in real life.

# References

| [1] | R. J. Smith, "Britannica," 28 Apr 2025. [Online]. Available: https://www.britannica.com/technology/engineering. [Accessed 8 May 2025]. |
| --- | --- |
| [2] | A. C. o. E. Companies, "ACEC," Yoko Co, 2025. [Online]. Available: https://www.acec.org/about-acec/what-is-engineering/. [Accessed 13 May 2025]. |
| [3] | N. University, "National University," 2025. [Online]. Available: https://www.nu.edu/blog/types-of-engineering/#15-different-types-of-engineering-jobs. [Accessed 13 May 2025]. |
| [4] | W. H. Organization, "World report on vision," World Health Organization, 2019. |
| [5] | "Arduino Store," [Online]. Available: https://store.arduino.cc/products/arduino-mega-2560-rev3. [Accessed May 2025]. |
| [6] | "Arduino Docs," [Online]. Available: https://docs.arduino.cc/hardware/mega-2560/. [Accessed May 2025]. |